ENPM808A FINAL PROJECT REPORT

# General Project Pipeline followed

My codebase includes following classes implementation:

- Data_import.py – Checks for csv in the input folder. Checks for any inconsisteCombines the data and make a new csv to avoid loading multiple csv’s again and again. Provides the data to the MPCLearning class present in pipeline.py module.

- Pipeline.py – This module takes care of the major part of the pipeline

The steps shown above in the flowchart are described below:

Data Gathering

In this project, we were already given the data, and hence has been assumed that the data is IID but with two minimum scenarios:

- A corridor scenario with moving obstacles Tested in a box scenario

- An open box/hall environment with moving obstacles

Since the sampling rate, sampling time along with sampling space may differ hence I decided to train for both scenarios separately.

Data Selection

A large number of CSV files are provided. The simplest thing to do was to take a random sub-sample with uniform distribution and check if it was significant or not. If it's reasonably significant, we'll keep it. If it's not, we'll take another sample and repeat the procedure until we get a good significance level. Initially, I considered just 1 CSV file with more than enough data to split in the training and validation just to set my pipeline. Once set, I added enough files to gather enough data points to follow the VC dimension rule. Well, it is always good to have a large amount of data but I kept in mind two things:

- Outliers

- Computational complexity

The outliers were removed through the process of data cleaning. I created a data import module that takes all the CSV files in the folder, combines them, and write a new CSV. Writing a CSV is time-consuming, but it has to happen only in the first run, and is very convenient to use the generated CSV again and again for tuning parameters. The same thing is done for testing CSV files.

Data Cleaning and Pre-processing

This is performed to check following flaws with the input data:

- Can be converted to NumPy float64 format for data type consistency?

- Are the number of rows consistent?

- Presence of any null value

- Repeated/Stagnant or Unchanging data

Source: https://www.geeksforgeeks.org/

Part of this process is handled by the data_import.py and pipeline.py module provided in the code base. The rest is taken care by the pre-processing tool of sklearn library. As mentioned on their page, the Standardization of datasets is a common requirement for many machine learning estimators implemented in scikit-learn; they might behave badly if the individual features do not more or less look like standard normally distributed data: Gaussian with zero mean and unit variance, basically white.

Feature Selection

There are three main goals to feature selection.

- Improve the accuracy with which the model can predict new data.

- Reduce computational cost.

- Produce a more interpretable model.

Through the given data I am trying to capture as much information as necessary for the controller The given data consists of Lidar data, robot position, local goal position, and final goal position. To define features and after knowing the scenario, I tried to think of both as a Controls engineer and as a Machine Learning engineer.

The features defined are as follows:

- Distance from the local goal position is one of the costs in MPC.

- Distance from the final goal position is one of the costs in MPC.

- Angular error in shortest trajectory to local goal and vehicle heading angle is another cost in MPC

Fig. Angular error between bot’s yaw and shortest trajectory

- Angular error in shortest trajectory to final goal and vehicle heading angle

- Lidar output in LOS (Line of Sight) of the robot is basically at the 540th index of lidar

- Lidar output towards the shortest trajectory for the local goal-

  - This was computed using the slope of the shortest trajectory and angular difference calculated above.

- Lidar output towards the shortest trajectory for the final goal

- Maximum distance the robot can travel towards the shortest trajectory for the local goal

Fig. Obstacle detection cost

Fig. Obstacle detection with tolerance in width of the bot (assuming 30 cm of width)

- Maximum distance the robot can travel towards the shortest trajectory for the final goal

Note: The limitation in output velocity and omega is also be considered

- Predicted velocity < Max bot velocity (found from given data)

- Predicted omega < Max bot omega (found from given data)

There can be more such features but these are the ones considered because of time limitations.

Metrics Considered

It represents the proportion of variance (of y) that has been explained by the independent variables in the model. It provides an indication of goodness of fit and therefore a measure of how well unseen samples are likely to be predicted by the model, through the proportion of explained variance.

As such variance is dataset dependent, R2 may not be meaningfully comparable across different datasets. Best possible score is 1.0 and it can be negative (because the model can be arbitrarily worse). A constant model that always predicts the expected (average) value of y, disregarding the input features, would get an R2 score of 0.0.

Note: when the prediction residuals have zero mean, the R2 score and the Explained variance score are identical.

If y^i is the predicted value of the i-th sample and yi is the corresponding true value for total n samples, the estimated R2 is defined as:

R2(y,y^)=1−∑i=1n(yi−y^i)2∑i=1n(yi−y¯)2

where y¯=1n∑i=1nyi and ∑i=1n(yi−y^i)2=∑i=1nϵi2.

SOURCE: https://scikit-learn.org/stable/modules/model_evaluation.html#regression-metrics

Model test with box data:

Linear Ridge Regression

Velocity Prediction:

Parameter Tuning for R2 score and MSE:

- R2 Score

- MSE variation

Large dataset

In-sample MSE = 0.046920975145010944

Validation MSE = 0.04698079021276492

Out-sample MSE = 0.051465936370142046

Learning Curve:

Scoring and error:

Omega Prediction:

Learning Curve:

Hyperparamter tuning:

R2 score

But testing score is in negative

MSE:

Large dataset omega

In-sample MSE = 0.06385259012498859

Validation MSE = 0.06472638682085763

Out-sample MSE = 0.06285082812775587

With 150,000 data points:

Velocity learning curve:

Omega Learning Curve:

SVM

The implementation is based on libsvm. The fit time scales at least quadratically with the number of samples and may be impractical beyond tens of thousands of samples. For large datasets consider using LinearSVC or SGDClassifier instead, possibly after a Nystroem transformer.

Hyperparameter tuning:

- RBF Kernel with poly degree 6:

Regularization:

Plotted against 1/alpha

R2 Score for Velocity = 0.483131221616796

MSE:

In-sample MSE = 0.044280547624567686

Validation MSE = 0.04472638682085763

Out-sample MSE = 0.04763863977038479

Learning Curve:

Omega:

R2 Score

MSE:

In-sample MSE = 0.05958532990772832

Validation MSE = 0.062082018320186366

Out-sample MSE = 0.06019050373651059

Learning Curve for Omega:

Outsample score = 0.04

- Linear Kernel: was not able to solve

- Poly Kernel: degree = 3, iterated through regularization,

Regularization: Plotted against 1/alpha

Velocity:

Learning Curve:

Outsample R2 score = 0.10818291218618581

Omega:

Learning curve:

- Sigmoid Kernel:

The output from these kernel was negative R2 scores which meant that the data is fitting very badly with the predicted weights.

XGBOOST (Source: NVIDIA.com):

XGBoost is an open-source software library that implements optimized distributed gradient boosting machine learning algorithms under the Gradient Boosting framework.

XGBoost, which stands for Extreme Gradient Boosting, is a scalable, distributed gradient-boosted  decision tree (GBDT) machine learning library. It provides parallel tree boosting and is the leading machine learning library for regression, classification, and ranking problems.  A Gradient Boosting Decision Trees (GBDT) is a decision tree ensemble learning algorithm similar to random forest, for classification and regression. Ensemble learning algorithms combine multiple machine learning algorithms to obtain a better model.

Random forest uses a technique called bagging to build full decision trees in parallel from random bootstrap samples of the data set. The final prediction is an average of all of the decision tree predictions.

The term “gradient boosting” comes from the idea of “boosting” or improving a single weak model by combining it with a number of other weak models in order to generate a collectively strong model. Gradient boosting is an extension of boosting where the process of additively generating weak models is formalized as a gradient descent algorithm over an objective function. Gradient boosting sets targeted outcomes for the next model in an effort to minimize errors. Targeted outcomes for each case are based on the gradient of the error (hence the name gradient boosting) with respect to the prediction.

Learning Curve:

Velocity->NEEDS MORE DATA POINTS

Omega:

Hyperparameter Tuning:

Velocity->

In-sample R2 score = 0.9928270626691554

Validation R2 score = 0.9624411385324139

Out-sample R2 score = 0.88950

Large Dataset

In-sample MSE = 0.001990886776502641

Validation MSE = 0.002237801974321346

Out-sample MSE = 0.013310271713501888

Omega:

R2 Score:

In-sample R2 score = 0.8807

Validation R2 score = 0.69446

Out-sample R2 score = 0.3016

MSE (Large dataset):

In-sample MSE = 0.007678344423281377

Validation MSE = 0.019899980335489773

Out-sample MSE = 0.04383341306052144

Velocity Learning curve

After increasing data points:

In-sample R2 score = 0.8807

Validation R2 score = 0.69446

Out-sample R2 score = 0.3016

reg:linear is now deprecated in favor of reg:squarederror.

[16:35:21] WARNING: C:/buildkite-agent/builds/buildkite-windows-cpu-autoscaling-group-i-030221e36e1a46bfb-1/xgboost/xgboost-ci-windows/src/objective/regression_obj.cu:213: reg:linear is now deprecated in favor of reg:squarederror.

NEURAL NETWORKS

Multi-layer Perceptron (Source: scikit-learn.org)

Multi-layer Perceptron (MLP) is a supervised learning algorithm that learns a function f(⋅):Rm → Ro by training on a dataset, where m is the number of dimensions for input and o is the number of dimensions for output. Given a set of features X=x1,x2,...,xm and a target y, it can learn a non-linear function approximator for either classification or regression. It is different from logistic regression, in that between the input and the output layer, there can be one or more non-linear layers, called hidden layers. Figure 1 shows a one hidden layer MLP with scalar output.

Regularisation and hyperparameter tuning:

With the max_iter parameter set to 500

Velocity:

In-sample R2 score = 0.6544640660728486

Validation R2 score = 0.5824411385324139

Out-sample R2 score = 0.5711991286745627

Omega:

In-sample R2 score = 0.0544

Validation R2 score = 0.0582

Out-sample R2 score = 0.052

Learning Curve:

Velocity

Omega:

After data scaling:

Velocity->

In-sample R2 score = 0.830

Validation R2 score = 0.835

Out-sample R2 score = 0.821

Omega->

In-sample R2 score = 0.2230

Validation R2 score = 0.2035

Out-sample R2 score = 0.1931

After Large dataset:

Velocity:

R2 score:

In-sample R2 score = 0.610

Validation R2 score = 0.615

Out-sample R2 score = 0.591

In-sample MSE = 0.04691647128188403

Validation R2 score = 0.04699860370372856

Out-sample R2 score = 0.019885021045387995

Omega:

In-sample R2 score = 0.038

Validation R2 score = 0.036

Out-sample R2 score = 0.03

In-sample MSE = 0.06203824884540909

Validation MSE = 0.06289030049585587

Out-sample MSE = 0.06952008743539813

Model test with corridor data:

Linear:

Velocity:

MSE:

In-sample MSE = 0.058763898617297965

Validation MSE = 0.05964313150902283

Out-sample MSE = 0.06604592908874049

Learning Curve:

Omega:

MSE

In-sample MSE = 0.04724216267807056

Validation MSE = 0.05964313150902283

Out-sample MSE = 0.0564443296827880

Learning Curve:

XG Boost

Velocity Prediction:

MSE:

In-sample MSE = 0. 002185511791884443

Validation MSE = 0.003195511791884443

Out-sample MSE = 0.03642986279028171